# Connecting with Android Auto

Android Auto enables a smartphone to project a driver-optimized interface onto the vehicle head unit while most logic continues to run on the phone.

## 1. How Android Auto Connection Works

### Basic Concept

- Apps run on the phone

- UI is projected to the car display

- Car acts as:

  - External screen

  - Audio device

  - Input controller

### Connection Requirements

- Android 8.0+ phone

- Compatible head unit

- USB cable or Wi-Fi

- Google Play Services

- Android Auto app/services

## 2. Using Android Auto APIs

Google provides the Android for Cars App Library instead of classic Android UI.

### Supported App Categories

- Media

- Navigation

- Messaging

- Charging

- Parking

### Core API Components

#### a. Car App Library

- Template-based UI

- Host renders interface

- App provides data & actions

#### b. Session & Screen Model

- CarAppService

- Session

- Screen

- Templates

#### c. Example Flow

- User launches app in car

- Host binds to CarAppService

- App returns templates

- Host renders UI

- User input sent back to phone

### Key Classes

- CarAppService

- Session

- ScreenManager

- Template

- Action

## 3. Communication Between Phone and Car

### Data Flow Architecture

Phone App  ←→  Android Auto Host  ←→  Car Head Unit

### What Is Shared

- Touch / rotary input

- Audio focus

- Microphone

- Display surface

- Sensor hints (day/night)

### What Is NOT Shared

- Direct ECU access

- Vehicle controls

- Sensitive car data

### Messaging Channels

- Binder IPC

- Google Play Services

- Media session

- Voice intents

### Latency Considerations

- UI rendered by car

- Logic executed on phone

- Requires efficient templates

## 4. Projection Modes

Android Auto supports two main modes:

### A. Wired Projection (USB)

#### Process

- Plug USB cable

- Head unit detects phone

- Authentication

- Projection session starts

#### Advantages

- Stable connection

- Low latency

- Charging phone

- Better audio quality

#### Limitations

- Cable required

- Port compatibility

### B. Wireless Projection

#### Technologies Used

- Wi-Fi Direct

- Bluetooth for discovery

- 5GHz communication

#### Steps

- Bluetooth pairing

- Wi-Fi negotiation

- Secure channel

- Projection starts

#### Pros

- Cable-free

- Convenience

- Automatic reconnect

#### Cons

- Higher battery use

- Possible interference

- Slight latency

## 5. Security Model

- User consent required

- App verification

- Restricted templates

- No background distraction

## 6. Developer Considerations

- Must use templates

- No custom views

- Voice support mandatory

- Driving restrictions applied