# Installing Apps on AAOS

Installing applications on AAOS is different from normal Android because the system runs inside the vehicle and must follow automotive safety and security rules.

## 1. Deployment Process on AAOS

### A. Development Phase

- Build app using:

  - Android Studio

  - Android for Cars App Library

  - AAOS SDK add-ons

- Test on:

  - AAOS Emulator

  - Desktop Head Unit (DHU)

  - Reference hardware

### B. Installation Methods

#### 1) Google Play Store in Vehicle

- Primary method for consumer apps

- Apps downloaded directly to car

- Automatic updates via OTA

- Region and vehicle filtering

#### 2) OEM Pre-Installation

- System apps (HVAC, camera, radio)

- Installed during manufacturing

- Signed with OEM platform keys

#### 3) ADB / Developer Install

- Used for testing

- adb install app.apk

- Requires developer mode

- Not allowed for production users

### C. Typical Deployment Flow

- Developer builds APK/AAB

- Local testing on emulator

- Compliance with automotive templates

- Upload to Play Console

- Automotive review process

- Release to vehicle Play Store

- OTA update lifecycle

## 2. Security Considerations

Security is stricter than phone Android because apps run inside a vehicle.

### A. Permission Model

- Special car permissions:

  - CAR_INFO

  - CAR_ENERGY

  - CAR_CONTROL (restricted)

- User consent required

- Driving-state restrictions

### B. Application Sandboxing

- Each app runs in isolated container

- No direct ECU access

- Communication only via:

  - Car Service

  - Vehicle HAL (for system apps)

### C. Driver Safety Rules

- Limited interaction while driving

- Template-based UI only

- No background distractions

- Voice-first design

### D. System Protection

- Verified boot

- Encrypted storage

- Secure OTA updates

- SELinux policies

## 3. App Signing and Distribution

### A. Signing Requirements

- All AAOS apps must be digitally signed

- Same Android signing model as mobile

- Two types:

- Play Store Apps

  - Signed with developer key

  - Google Play App Signing

  - Automotive approval required

- System/OEM Apps

  - Platform keys

  - Higher privileges

  - Access to vehicle features

### B. Distribution Channels

| Channel | Purpose |
| --- | --- |
| Google Play Automotive | Third-party apps |
| OEM App Store | Brand specific apps |
| Factory Image | System applications |
| Service Center | Maintenance install |

### C. Update Mechanisms

- Play Store auto update

- Vehicle OTA packages

- Incremental updates

- Rollback protection

## 4. Differences from Phone Android

| Aspect | Phone Android | AAOS |
| --- | --- | --- |
| Install source | Phone Play Store | Car Play Store |
| Hardware access | Phone sensors | Vehicle HAL |
| UI | Custom allowed | Templates only |
| Safety limits | Few | Strict |
| Signing | Standard | Automotive review |