# Introduction to Android Automotive OS (AAOS)

## 1. What is AAOS?

Android Automotive OS (AAOS) is a full Android-based operating system designed to run directly on vehicle hardware as the car’s primary infotainment platform.

- It is not just projection from a phone

- The car itself runs Android

- Apps can access vehicle hardware through secure car APIs

### Goals of AAOS

- Provide a modern app ecosystem inside vehicles

- Standardize infotainment software across manufacturers

- Enable OTA updates and Google services

- Improve safety with driver-optimized UI

### Examples of AAOS Vehicles

- Volvo / Polestar

- Renault

- GM vehicles with Google built-in

- Honda, Ford (new generations)

## 2. AAOS vs Android Auto – Don’t Confuse Them!

| Feature | Android Auto | Android Automotive OS |
| --- | --- | --- |
| Runs on | User’s phone | Vehicle hardware |
| Needs phone | Yes | No |
| App installation | From phone | From Play Store in car |
| Access to vehicle data | Limited | Direct via Vehicle HAL |
| Offline use | No | Yes |

# 3. AAOS Architecture and Components

AAOS builds on standard Android but adds car-specific layers.

## High-Level Stack

+--------------------------------------------------+

| Applications (Media, Navigation, HVAC, Phone)    |

+--------------------------------------------------+

| Car APIs & Car Services                          |

+--------------------------------------------------+

| Android Framework                                |

+--------------------------------------------------+

| HAL (Vehicle Hardware Abstraction Layer)         |

+--------------------------------------------------+

| Vehicle ECUs & Hardware                          |

+--------------------------------------------------+

## 3.1 Major Components

### A. System UI

- Car-optimized launcher

- Notification center

- Quick controls (HVAC, volume, drive modes)

### B. Car Service (CarService)

Central system service that:

- Exposes vehicle features to apps

- Manages permissions

- Communicates with Vehicle HAL

### C. Vehicle HAL (VHAL)

Bridge between Android and car electronics

- Reads vehicle signals

- Controls hardware like:

  - Climate

  - Doors

  - Lights

  - Speed

  - Battery

### D. Car API

Used by developers:

- CarPropertyManager

- CarAudioManager

- CarSensorManager

- CarNavigationStatusManager

## 3.2 Boot Process (Simplified)

- Vehicle powers on

- Android kernel boots

- System services start

- CarService connects to VHAL

- System UI launcher appears

# 4. Key Differences: AAOS vs Standard Android

## 4.1 UI / UX Differences

- Driver distraction rules

- Large touch targets

- Limited text input while driving

- Rotary controller support

- Voice-first interaction

## 4.2 Permissions & Security

- Car permissions:

  - CONTROL_CAR_DOORS

  - CAR_SPEED

  - HVAC_CONTROL

- Apps are restricted by:

  - Driving state

  - User profiles

  - OEM policies

## 4.3 App Types

### Allowed categories:

- Media

- Navigation

- Charging

- Parking

- Messaging (limited)

### Not allowed:

- Games while driving

- Complex text entry

- Distracting animations

## 4.4 Hardware Access

Standard Android:

- Camera, GPS, sensors from phone

AAOS:

- Vehicle signals

- CAN bus data

- ECU communication

# 5. Use Cases & Target Devices

## 5.1 Infotainment

- Music & podcasts

- Radio integration

- Video (parked mode)

## 5.2 Navigation

- Google Maps

- OEM navigation

- EV range routing

## 5.3 Vehicle Controls

- Climate

- Seat position

- Drive modes

- ADAS settings

## 5.4 EV Features

- Battery status

- Charging stations

- Preconditioning

## 5.5 Fleet & Commercial

- Taxis

- Delivery vehicles

- Trucks

- Shared mobility

# 6. Development Overview

## App Development

- Same Android SDK

- Special car libraries

- Templates for:

  - Media

  - Navigation

  - Messaging

## Testing Tools

- AAOS Emulator

- Desktop Head Unit

- Vehicle Hardware rigs

# 7. Advantages of AAOS

- OTA updates

- Unified app ecosystem

- No phone dependency

- Deep vehicle integration

- Multi-user profiles

## Challenges

- OEM customization

- Safety certification

- Long automotive lifecycle

- Limited app categories