AUTOMOTIVE ELECTRONICS – COMPLETE STUDY MATERIAL

CHAPTER 1: INTRODUCTION TO AUTOMOTIVE ELECTRONICS

Modern vehicles are no longer mechanical machines only.
They are computer-controlled systems with advanced electronics that improve:

- Safety

- Fuel efficiency

- Comfort

- Emission control

- Automation

At the heart of this system are ECUs, communication networks, and embedded software.

CHAPTER 2: ELECTRONIC CONTROL UNIT (ECU)

2.1 What is an ECU?

An ECU (Electronic Control Unit) is a small embedded computer installed in a vehicle that controls one or more electrical systems.

A modern car contains 70 to 150 ECUs.

2.2 Types of ECUs

| ECU Name | Function |
| --- | --- |
| Engine Control Unit (ECU) | Controls fuel injection, ignition |
| Transmission ECU | Controls gear shifting |
| Brake ECU (ABS/ESP) | Controls braking and stability |
| Body Control Module (BCM) | Controls lights, doors, windows |
| Airbag ECU | Controls airbag deployment |
| Climate Control ECU | Controls AC & heating |
| Infotainment ECU | Controls multimedia & navigation |

2.3 Internal Structure of an ECU

An ECU consists of:

- Microcontroller (CPU)

- Memory – ROM, RAM, EEPROM

- Input Interfaces – Sensors

- Output Interfaces – Actuators

- Communication Interface – CAN, LIN, Flex Ray, Ethernet

- Power Supply Circuit

2.4 Working of an ECU

Sensors → ECU Processing → Actuators

Example:
Speed sensor → ECU calculates → Brake pressure adjusted

CHAPTER 3: CAN BUS (Controller Area Network)

3.1 Introduction

CAN is a communication system that allows ECUs to exchange data with each other in real time.

It reduces wiring complexity and increases reliability.

3.2 Features of CAN

- Multi-master system

- Message based protocol

- Error detection & correction

- High noise immunity

- Real-time communication

- Low wiring complexity

3.3 CAN Physical Layer

Uses two wires:

- CAN High (CAN_H)

- CAN Low (CAN_L)

Differential signalling improves noise resistance.

3.4 CAN Frame Format

| Field | Description |
| --- | --- |
| SOF | Start of Frame |
| Identifier | Message priority |
| Control Field | Frame information |
| Data Field | 0–8 bytes |
| CRC | Error check |
| ACK | Acknowledgment |
| EOF | End of Frame |

3.5 CAN Arbitration

When two ECUs transmit simultaneously:

- ECU with lower ID wins

- Other ECUs stop transmitting

CHAPTER 4: AUTOMOTIVE NETWORK TOPOLOGIES

4.1 What is Network Topology?

It is the physical arrangement of ECUs and communication networks.

4.2 Types of Topologies

| Topology | Explanation |
| --- | --- |
| Bus | All ECUs connected to a common line |
| Star | All ECUs connected to central node |
| Ring | Each ECU connected in a loop |
| Hybrid | Combination of above |

4.3 In-Vehicle Networks

| Network | Application |
| --- | --- |
| CAN | Powertrain, chassis |
| LIN | Body electronics |
| FlexRay | Safety critical |
| Ethernet | ADAS, camera systems |

CHAPTER 5: EMBEDDED SOFTWARE ARCHITECTURE

5.1 What is Embedded Software?

Software programmed inside ECU to control vehicle functions.

5.2 AUTOSAR Layered Architecture

Application Layer

--------------------

Runtime Environment (RTE)

--------------------

Basic Software (BSW)

--------------------

MCAL

--------------------

Hardware

5.3 Explanation of Layers

Application Layer

Contains vehicle functions.

RTE

Connects software components.

Basic Software

Provides communication, diagnostics, OS, memory services.

MCAL

Hardware abstraction.

CHAPTER 6: COMPLETE SYSTEM WORKING

Sensor → ECU → CAN Bus → Other ECUs → Actuator

Example:
Brake pedal → Brake ECU → CAN → Engine ECU → Reduce torque

CHAPTER 7: ADVANTAGES OF AUTOMOTIVE ELECTRONICS

- Better fuel economy

- Increased safety

- Reduced emissions

- Improved comfort

- Advanced diagnostics

- Autonomous driving support

CHAPTER 8: SHORT NOTES FOR EXAM

ECU

Small embedded computer controlling vehicle systems.

CAN Bus

Main communication network of vehicle.

Network Topology

Arrangement of ECUs and communication links.

Embedded Software

Programs that control ECU operations.