AUTOSAR – Complete Study Material

1. What is AUTOSAR?

AUTOSAR (Automotive Open System Architecture) is a standardized software architecture for automotive ECUs that:

- Enables reusable software

- Supports supplier independence

- Improves scalability and maintainability

Enables Reusable Software

Without AUTOSAR, software is often tightly coupled to specific hardware and ECU designs.
With AUTOSAR:

- Application software is hardware-independent

- Software Components (SWCs) can be reused across different ECUs and vehicle models

- The same software can be deployed with different microcontrollers by only changing the lower layers (BSW/MCAL)

Result:
Development becomes faster, cheaper, and more reliable because existing software can be reused.

Supports Supplier Independence

Traditionally, OEMs were locked into specific suppliers because their software and hardware were tightly integrated.

AUTOSAR solves this by defining:

- Standard interfaces between software layers

- Common communication mechanisms

- Well-defined APIs for services

This allows:

- One supplier to develop the application

- Another supplier to provide BSW

- Another supplier to provide hardware

All components still work together because they follow the same AUTOSAR standard.

2. Why AUTOSAR?

| Problem | AUTOSAR Solution |
| --- | --- |
| Vendor lock-in | Standardized interfaces |
| High development cost | Reusable software components |
| Complex integration | Layered architecture |

2.AUTOSAR Layered Architecture

+-------------------------+

|     Application Layer |

+-------------------------+

|     RTE (Middleware)   |

+-------------------------+

|   Basic Software (BSW) |

+-------------------------+

|     Microcontroller    |

+-------------------------+

Layer Responsibilities

A. Application Layer

- Contains Software Components (SWCs)

- Hardware independent

- Implements vehicle functionality

B. RTE (Run-Time Environment)

- Middleware

- Connects SWCs with BSW

- Enables communication between components

C. Basic Software (BSW)

Provides services for:

- Communication

- Memory

- OS

- Diagnostics

- Hardware abstraction

D. Microcontroller Layer

- Hardware drivers

- ECU peripherals

3.Basic Software Modules (BSW)

BSW Structure

BSW

├── Services Layer

├── ECU Abstraction Layer

├── Microcontroller Abstraction Layer (MCAL)

└── Complex Drivers

Key Modules

| Layer | Example Modules |
| --- | --- |
| Services | OS, COM, DCM, NVM |
| ECU Abstraction | I/O HW Abstraction |
| MCAL | ADC, CAN, SPI, PWM |
| Complex Drivers | Special hardware drivers |

Example:

COM Module handles:

- Signal transmission

- Message packing

- Communication scheduling

4. RTE Concepts

Purpose of RTE

- Provides communication between SWCs and BSW

- Hides hardware and BSW details from application

Key Concepts

| Concept | Explanation |
| --- | --- |
| Ports | Sender-Receiver, Client-Server |
| Interfaces | Data type definitions |
| Runnable | Executable function in SWC |
| Events | Trigger runnable |

Example Flow

SWC → RTE → COM → CAN Driver → Bus

5.Configuring Basic AUTOSAR Components

Configuration Steps

- Create System Description (.arxml)

The. arxml file is the main configuration file in AUTOSAR.

It contains:

- ECU information

- Software components

- Communication definitions

- Network topology

This file acts as the blueprint of the whole ECU software.

- Define Software Components (SWCs)

SWCs represent the application functions of the ECU.

Examples:

- Engine control

- Door control

- Brake management

Each SWC contains:

- Runnable (functions)

- Input/Output ports

- Events

- Define Ports & Interfaces

Ports describe how SWCs communicate.

Two main communication types:

| Type | Purpose |
| --- | --- |
| Sender-Receiver | Data exchange |
| Client-Server | Service requests |

Interfaces define:

- Data types

- Communication rules

This ensures that SWCs can interact correctly.

- Configure BSW Modules

Basic Software (BSW) provides essential services.

Important modules to configure:

| Module | Purpose |
| --- | --- |
| OS | Task scheduling and resource management |
| COM | Signal & message communication |
| CAN Stack | Handles CAN bus communication |
| NVM | Manages non-volatile memory |
| ECU Manager (ECUM) | Controls ECU startup and shutdown |
| Watchdog (Wdg) | Detects software faults |

Each module is configured using parameters defined in AUTOSAR tools.

- Generate RTE Code

The RTE (Run-Time Environment) is automatically generated from the configuration.

It:

- Connects SWCs with each other

- Connects SWCs to BSW

- Provides standardized APIs

This code acts as the middleware.

- Compile and Integrate

Finally:

- All generated code is compiled

- Linked with application and BSW code

- Flashed into the ECU

- Tested on real hardware

6.AUTOSAR Development Environment Setup

Required Tools

| Tool | Purpose |
| --- | --- |
| Eclipse IDE | Development platform |
| AUTOSAR Plugin | Model & configuration |
| Compiler | Build |
| MCAL package | Hardware drivers |
| RTE Generator | Middleware generation |

Basic Setup Procedure

- Install Eclipse

- Install AUTOSAR plugin (Artop / Vendor tool)

- Import ECU project

- Configure SWCs & BSW

- Generate RTE

- Build and flash ECU

7. Example Configuration Workflow

Requirements → SWC Design → BSW Config → RTE Generation → Build → Test