Network Diagnostics – AUTOSAR Concepts

1. Introduction

In modern vehicles, many ECUs communicate over networks such as CAN, LIN, Flex Ray, and Ethernet.
Network diagnostics ensures that this communication remains reliable, detectable, and recoverable when faults occur.

AUTOSAR provides standardized diagnostic and network management modules to:

- Monitor ECU health

- Detect faults

- Store error information

- Support vehicle servicing

2. Diagnostic Manager (DEM – Diagnostic Event Manager)

The Diagnostic Manager (DEM) is the core AUTOSAR module responsible for fault detection and storage.

Functions of DEM

| Function | Description |
| --- | --- |
| Event detection | Detects software & hardware faults |
| DTC management | Stores and manages Diagnostic Trouble Codes |
| Status tracking | Monitors event status (active, confirmed, stored) |
| Memory handling | Stores fault info in non-volatile memory |

Example

If a CAN message from the brake ECU is missing:

- DEM detects communication failure

- Stores a DTC

- Makes fault information available for diagnostics

3. Network Management (NM) Services

Network Management ensures that ECUs share the bus efficiently and only remain active when needed.

Functions of NM

| Function | Purpose |
| --- | --- |
| Wake-up & sleep | Controls ECU power states |
| Bus coordination | Synchronizes ECU communication |
| Network supervision | Detects missing nodes |
| Energy efficiency | Reduces battery consumption |

Types of NM

- CAN NM

- LIN NM

- Flex Ray NM

- Ethernet NM

Example

When the vehicle is locked:

- NM transitions ECUs into sleep mode

- Only essential ECUs remain active

- Network load and power usage drop

4. Diagnostic Trouble Codes (DTCs)

A DTC is a standardized code representing a specific fault.

DTC Structure

| Part | Meaning |
| --- | --- |
| System | Powertrain, Body, Chassis, Network |
| Failure type | Specific fault category |

Purpose of DTCs

- Identify faults quickly

- Support service technicians

- Enable fault history tracking

Example DTC

U0100 – Lost communication with Engine Control Module

5. How AUTOSAR Diagnostics Work Together

Fault Occurs → DEM Detects → DTC Stored → DCM Reports → Service Tool Reads

Diagnostic Communication Manager (DCM)

DCM handles:

- Diagnostic requests from service tools

- Communication of DTCs and ECU data

- UDS services (Read DTC, Clear DTC, etc.)

Why Network Diagnostics Matter

| Benefit | Explanation |
| --- | --- |
| Fault isolation | Quickly locate problem ECUs |
| System reliability | Prevent cascading failures |
| Vehicle safety | Detect critical faults early |
| Maintenance efficiency | Faster vehicle servicing |

Introduction to CAN Bus Message Analysis

The CAN (Controller Area Network) bus is the primary communication system used in modern vehicles.
It allows different Electronic Control Units (ECUs)—such as the engine ECU, brake ECU, body control module, and others—to exchange information quickly and reliably over a shared network.

Because so many critical vehicle functions depend on this communication, engineers must constantly analyze CAN messages to ensure the network is working correctly.

Analysing CAN messages helps engineers in the following ways:

1. Detect Communication Errors

CAN analysis allows engineers to identify:

- Missing messages

- Corrupted data frames

- Bus errors such as CRC errors, bit errors, or bus-off conditions

These errors can cause ECUs to lose synchronization or stop communicating, leading to serious vehicle malfunctions.

2. Diagnose ECU Problems

When an ECU fails or behaves abnormally, CAN analysis can show:

- Whether the ECU is transmitting incorrect data

- Whether it is responding to requests

- Whether it has stopped communicating completely

This helps engineers locate faulty ECUs, wiring issues, or software bugs.

3. Verify Correct Signal Transmission

Each CAN message contains signals (such as speed, temperature, door status).
By decoding these signals, engineers can verify that:

- The correct data is being sent

- The data values are within expected limits

- The timing of messages matches system requirements

This ensures that vehicle functions operate as designed.

4. Ensure System Reliability

Regular CAN analysis ensures:

- Stable communication between ECUs

- Early detection of potential failures

- Safe and predictable vehicle behavior

This increases overall vehicle reliability and safety.

2. Why Analyse CAN Messages?

CAN message analysis is done to:

| Purpose | Explanation |
| --- | --- |
| Fault detection | Identify missing or corrupted messages |
| Debugging | Find wrong signal values |
| Validation | Check correct network behaviour |
| Diagnostics | Support vehicle fault diagnosis |

3. CAN Frame Structure

| SOF | ID | RTR | DLC | DATA | CRC | ACK | EOF |

Important fields for diagnostics:

1. ID (Identifier) – Identifies Message Type

The ID defines:

- What kind of message it is

- Which ECU sent it

- How important it is (priority)

For diagnostics:

- Engineers use the ID to recognize specific messages such as speed, engine status, brake status, etc.

- If an expected ID is missing, it indicates a communication or ECU fault.

2. DLC (Data Length Code) – Data Size

The DLC specifies how many data bytes are in the message (0 to 8 bytes).

For diagnostics:

- A wrong DLC value can indicate configuration errors or corrupted frames.

- It helps ensure the receiving ECU reads the correct amount of data.

3. DATA – Actual Signal Values

The DATA field carries the actual information such as:

- Vehicle speed

- Engine temperature

- Door status

- Sensor readings

For diagnostics:

- Engineers decode these bytes into signals and check whether the values are correct.

- Incorrect data values point to sensor issues, software bugs, or communication faults.

4. CRC (Cyclic Redundancy Check) – Error Detection

The CRC field is used to detect transmission errors.

For diagnostics:

- If CRC errors increase, it indicates noise, wiring problems, or faulty hardware.

- CRC ensures that corrupted messages are not used by the ECUs.

4. CAN Bus Analyzer – What It Does

A CAN bus analyser is a hardware + software tool used to:

- Capture CAN messages

- Display message ID, data, timing

- Detect errors (bit error, stuff error, CRC error)

- Monitor bus load and performance

Common Capabilities

- Real-time message logging

- Signal decoding using DBC file

- Error frame detection

- Message filtering

5. Diagnostic Use Case Example

Problem:

Vehicle door not locking properly.

Step 1: Capture CAN Traffic

Using a CAN bus analyser, engineers record all messages being transmitted on the CAN network.

This provides a live view of:

- Message IDs

- Timing

- Data values

- Error frames

This step creates the diagnostic baseline.

Step 2: Observe Door Control Message ID

Each function has a specific CAN message ID.

Engineers identify the message ID responsible for door locking, for example:

- ID 0x245 → Door Lock Command

If this ID is missing or not transmitted at the expected rate, it indicates a communication or ECU issue.

Step 3: Check Signal Values (Lock/Unlock Status)

Inside the DATA field, engineers decode the lock/unlock signal.

They verify:

- Whether the lock command is actually sent

- Whether the signal value changes when the lock button is pressed

- Whether the receiving ECU interprets it correctly

Incorrect signal values point to software or sensor faults.

Step 4: Detect Missing or Delayed Messages

Engineers examine message timing:

- Missing messages → ECU stopped transmitting

- Delayed messages → ECU overloaded or network congestion

Both conditions can prevent the door from responding correctly.

Step 5: Identify Faulty ECU or Wiring

After analysing communication:

- If messages are correct → check actuators or wiring

- If messages are missing or wrong → investigate the transmitting ECU

- If many errors appear → inspect physical wiring or termination

This systematic approach allows accurate fault isolation.

6. AUTOSAR Diagnostics Architecture

AUTOSAR provides standardized diagnostic services:

Application

↓

RTE

↓

DCM (Diagnostic Communication Manager)

↓

DEM (Diagnostic Event Manager)

↓

BSW → CAN Driver

Important AUTOSAR Diagnostic Modules

| Module | Function |
| --- | --- |
| DCM | Handles diagnostic requests (UDS services) |
| DEM | Stores fault events & DTCs |
| NVM | Stores diagnostic memory |
| COM | Transfers diagnostic messages |
| CAN Stack | Sends/receives CAN frames |

7. Troubleshooting Communication Issues

Common CAN Problems

| Issue | Cause |
| --- | --- |
| No communication | Wiring fault, ECU power issue |
| Bus off | Too many errors |
| Missing frames | ECU malfunction |
| Wrong signal values | Software bug or configuration error |
| High bus load | Overloaded network |

Troubleshooting Process

- Capture CAN log

- Analyse message frequency & IDs

- Check error counters

- Validate signal values

- Compare with expected behaviour

8. Diagnostic Services (UDS Examples)

| Service | Purpose |
| --- | --- |
| Read DTCs | View stored faults |
| Clear DTCs | Reset fault memory |
| Read Data by ID | Retrieve sensor values |
| Routine Control | Execute tests |