Android for Automotive Infotainment Systems

1. Introduction

Android Automotive OS (AAOS) is Google’s version of Android designed specifically for in-vehicle infotainment systems.
Unlike Android Auto (which runs on a phone), AAOS runs directly on the vehicle’s head unit.

It enables OEMs to build modern infotainment systems using Android while integrating deeply with vehicle hardware and AUTOSAR services.

2. Android Automotive OS – Key Features

| Feature | Description |
| --- | --- |
| Native vehicle OS | Runs directly on ECU/head unit |
| Optimized UI | Designed for driving conditions |
| Multi-display support | Cluster + center display |
| Vehicle HAL (VHAL) | Interface to vehicle hardware |
| App ecosystem | Supports navigation, media, voice apps |
| OTA updates | Over-the-air software updates |

3. Head-Unit Applications

Head-unit apps are the main applications running on the infotainment system.

Common Head-Unit App Types

- Navigation app

- Media player

- Phone & messaging

- Vehicle settings

- Climate control UI

App Architecture

UI Layer → App Logic → Android Framework → Vehicle HAL → ECU Hardware

4. Integration with Vehicle Hardware

AAOS accesses vehicle data through Vehicle HAL (VHAL).

VHAL communicates with:

- AUTOSAR BSW

- Vehicle sensors

- ECUs

Example Data

- Speed

- Gear position

- Climate status

- Battery level

- Door status

5. Developing Infotainment Apps Using AAOS

Tools Required

- Android Studio

- Android Automotive SDK

- AAF (Android Automotive Framework) libraries

- Emulator or AAOS hardware target

Basic Development Steps

- Set up Android Studio with AAOS

- Create Automotive app project

- Design driving-optimized UI

- Access vehicle data via VHAL

- Test using emulator/vehicle target

6. Integrating with AUTOSAR Services

AAOS apps communicate with vehicle services through:

Android App → VHAL → Middleware → AUTOSAR BSW → ECU

Example Integration

- Read speed → AUTOSAR COM → CAN

- Set climate → AUTOSAR services → HVAC ECU

- Receive diagnostics → AUTOSAR DCM/DEM