AUTOSAR for Infotainment Systems

1. Introduction: Role of AUTOSAR in Infotainment

Modern infotainment systems require high reliability, safety, and standardization because they interact with many ECUs and vehicle networks.

AUTOSAR provides:

- A standardized software architecture

- Reusable services for communication, diagnostics, and memory

- Safe interaction between infotainment applications and vehicle hardware

It acts as the foundation layer between vehicle electronics and infotainment applications such as navigation, media, and HMI.

2. AUTOSAR Services for Infotainment

AUTOSAR provides core services that infotainment systems depend on.

A. AUTOSAR Services for HMI

| AUTOSAR Module | Role |
| --- | --- |
| RTE | Connects HMI software with BSW services |
| COM | Transfers HMI data over CAN/Ethernet |
| OS | Task scheduling for HMI applications |
| ECUM | Controls startup/shutdown of infotainment ECU |

B. Media Streaming Services

| Service | Purpose |
| --- | --- |
| COM | Transfers audio/video control signals |
| PduR | Routes multimedia data |
| EthIf | Ethernet communication for streaming |
| NVM | Stores media preferences & settings |

C. Navigation Data Management

| Module | Role |
| --- | --- |
| NVM | Stores map data & user settings |
| COM | Exchanges GPS and navigation messages |
| DCM | Diagnostics & updates |
| SoAd | Socket adapter for online services |

3. AUTOSAR Architecture in Infotainment

Android / Infotainment Apps

↓

Middleware / VHAL

↓

RTE (AUTOSAR)

↓

BSW Services

↓

MCAL Drivers

↓

Vehicle Hardware

4. Configuring AUTOSAR for Infotainment

Step-by-Step Configuration

- Create ECU & System description (.arxml)

- Configure BSW modules:

  - COM, PduR, OS, ECUM, DCM, NVM, EthIf

- Define SWCs for:

  - HMI control

  - Media management

  - Navigation interface

- Generate RTE

- Integrate & test

Key Tools

- Eclipse IDE

- AUTOSAR plugins (Artop, vendor tools)

5. Communication Between AUTOSAR and Android

Integration Flow

Android App → VHAL → Middleware → RTE → AUTOSAR COM → CAN/Ethernet → ECUs

Example Use Case: Volume Control

- User presses volume button on Android UI

- Android app sends command through VHAL

- Middleware translates to AUTOSAR signal

- COM module transmits via CAN

- Amplifier ECU receives and adjusts volume

6. Why AUTOSAR is Critical for Infotainment

| Benefit | Explanation |
| --- | --- |
| Standardization | Uniform software design |
| Safety | Isolates critical systems |
| Scalability | Easy upgrades |
| Interoperability | Supports Android integration |
| Maintainability | Simplifies long-term support |