1. Introduction to MNO Certification for Automotive Applications

What is MNO Certification?

MNO (Mobile Network Operator) Certification is the approval process required by a cellular operator (e.g., Vodafone, AT&T, Deutsche Telekom) before a device is allowed to operate on their live network.

In automotive applications, this ensures the vehicle’s connectivity module is:

Network-compatible

Secure

Reliable over long lifecycles (10–15 years)

Safe for critical vehicle functions

Why MNO Certification is Critical in Automotive?

Vehicles use cellular connectivity for:

Telematics Control Units (TCU)

eCall systems

OTA software updates

Remote diagnostics

Vehicle tracking and fleet management

A non-certified device may:

Be blocked from the network

Cause network instability

Create cybersecurity risks

Fail regulatory audits

2. Cellular Network Compliance Requirements

2.1 Standards Compliance

Automotive cellular devices must comply with global standards defined by:

3GPP – Defines LTE, LTE-M, NB-IoT, and 5G specifications

GSMA – Defines operator and SIM requirements

Key standards:

LTE Cat-1 / Cat-4 / Cat-M / NB-IoT

5G NR (for advanced automotive use cases)

VoLTE (mandatory for eCall in many regions)

2.2 Radio Frequency (RF) Compliance

Transmit power limits

Receiver sensitivity

Band support per region

Antenna performance

Coexistence with GNSS, Bluetooth, Wi-Fi

2.3 Protocol & Network Behaviour Compliance

Attach and detach procedures

Handover between cells

Idle and connected mode behaviour

Network recovery after signal loss

Power saving mechanisms (PSM, eDRX)

3. Security Requirements for Automotive MNO Certification

3.1 SIM & Subscription Security

Physical SIM (pSIM), eSIM, or iSIM

Secure storage of IMSI and keys

Proper authentication with network (AKA)

3.2 Device Security

Secure boot chain

Hardware Root of Trust

Encrypted firmware and storage

Protection against:

SIM cloning

Network spoofing

Unauthorized firmware changes

3.3 Network Security Compliance

TLS/IPSec usage for backend communication

Secure APN access

Firewall and port control

Protection against rogue base stations

3.4 Automotive-Specific Security Expectations

Compliance with ISO 21434 (road vehicle cybersecurity)

Separation between infotainment and safety-critical ECUs

Secure logging and traceability

4. Over-the-Air (OTA) Updates Requirements

4

4.1 Why OTA is Mandatory

Vehicles cannot rely on service visits for software updates

Security patches and feature updates must be remote

Regulatory and safety updates may be legally required

4.2 OTA Technical Requirements

Delta and full firmware updates

Resume capability after interruption

Rollback mechanism in case of failure

Update integrity verification (hash, signature)

4.3 MNO Expectations for OTA

Controlled data usage

Network-friendly download behaviour

No excessive signalling load

Support for large file transfers

5. MNO Certification Process – Step by Step

4

Step 1: Pre-Certification Preparation

Select target MNOs and regions

Review MNO certification documents

Confirm supported frequency bands

Freeze hardware design

Step 2: Conformance Testing

RF tests (3GPP)

Protocol stack testing

SIM and network attach tests

Conducted in certified labs

Step 3: Interoperability Testing (IOT)

Tests with live or simulated MNO network

Validates real-world behaviour

Focus on roaming and handover

Step 4: Field Testing

Drive tests in live networks

Urban, rural, and border scenarios

Power cycle and long-duration tests

Step 5: Certification Approval

Submission of test reports

Issue resolution if failures occur

Final MNO approval letter

6. Preparing a Device for MNO Certification Testing

Hardware Readiness

Production-level modem and antenna

Stable power supply

Automotive temperature support

Software Readiness

Stable modem firmware

Logging enabled

Network parameters configurable

OTA client integrated

Documentation Preparation

Device description

RF band support matrix

Software version details

Test configuration files

7. MNO Specific Tools and Test Procedures

Common Test Tools

Network simulators (Keysight, Rohde & Schwarz)

Protocol analysers

RF chambers

Drive test tools

MNO-Specific Test Areas

Network attach timing

Emergency call (eCall) behaviour

Data session recovery

SIM provisioning

Power consumption in idle state

8. Analysing Compliance Reports & Fixing Certification Issues

Common Certification Failures

Network attach failures

Excessive signalling

RF out-of-limit results

SIM authentication errors

OTA interruptions

How to Analyse Test Reports

Identify failed test cases

Check logs and traces

Correlate failures with network events

Validate against 3GPP specs

Issue Resolution Strategy

Firmware optimization

Modem parameter tuning

Antenna redesign if required

Re-testing and re-submission

9. Key Takeaways (Exam & Interview Ready)

MNO certification is mandatory for automotive cellular devices

Covers network compliance, security, and OTA

Automotive certification is stricter than consumer devices

Early preparation reduces certification delays

OTA and cybersecurity are top priorities for MNOs.